Preprocessing

- I’m able to load a: ppd file & CSV of behavioral annotations.

  - I still need to be able to upload the matching index.csv

- I’m able to extract the ID and date from the index

- I’m able to crop the data, deciding in which second to start and finish reading it

- Preprocessing code from PyPhotometry contains: denoising, low & high pass filtering, photobleaching correction, fitting a double exponent to the curve and subtracting it, motion correction, calculation of df/f and z-score.

- Create a .pkl file with: All the relevant GCaMP data, behavioral annotations, mouse index

Postprocessing

- Print the entire experiment’s df/f + behavioral data

- Zoom in every time point I want

- For calculating events: 1. Calc threshold, need to get baseline avg. based on “get_gcamp_data” (choose relevant frames manually (sec*30fps)). 2. Work with already calculated z-score, decide SD-related threshold based on visualization.